PROJECT DOCUMENTATION

Groundwater Level Depletion Forecasting

AI-powered prediction & risk classification for Western Uttar Pradesh

1M1B AI for Sustainability Virtual Internship

IBM SkillsBuild & AICTE

| Student Name | Badal Kumar |
| --- | --- |
| University | Quantum University, Roorkee, Uttarakhand |
| Email | badalkumar88935@gmail.com |
| SDG Alignment | SDG 6 — Clean Water & Sanitation ¦ SDG 13 — Climate Action |
| Data Source | Central Ground Water Board (CGWB), 1996–2017 |
| AI Technique | Random Forest (lag features) + Linear Trend Analysis |
| Study Area | Saharanpur, Muzaffarnagar, Meerut, Bijnor, Moradabad, Bareilly (UP) |
| Document Date | June 2026 |

# 1. Introduction

## 1.1 Background

Groundwater is India's most critical water resource, supplying approximately 60% of all irrigation demand and over 85% of rural drinking water. India is the world's third-largest groundwater consumer, and several states — including Uttar Pradesh — have been classified as having critically over-exploited groundwater zones by the Central Ground Water Board (CGWB).

Western Uttar Pradesh, encompassing districts like Saharanpur, Muzaffarnagar, Meerut, Bijnor, Moradabad, and Bareilly, has experienced significant groundwater depletion over the past two decades due to intensive agriculture, rapid urbanisation, and erratic monsoon recharge. Yet no accessible early-warning or forecasting tool exists at the district level for these communities.

## 1.2 Problem Statement

How might we use AI to forecast groundwater depletion so that communities in Western Uttar Pradesh can plan water use more sustainably?

This project addresses this question by building a machine-learning forecasting tool trained on two decades of real CGWB well-depth data, producing 3-year district-level predictions alongside interpretable risk classifications.

## 1.3 Objectives

- Analyse historical groundwater level trends (1996–2017) across 6 Western UP districts

- Build a Random Forest prediction model with lag features and seasonal indicators

- Produce a 3-year forecast of groundwater depth per district

- Classify each district's depletion risk as Low, Moderate, or High

- Deliver a self-contained offline web dashboard for visual exploration

# 2. SDG Alignment

This project directly contributes to two United Nations Sustainable Development Goals:

## 2.1 SDG 6 — Clean Water and Sanitation

- Forecasts groundwater depth trends to support sustainable water resource management

- Classifies depletion risk at the district level for informed extraction planning

- Provides a data-driven basis for groundwater conservation interventions

- Built entirely on 20+ years of verified public CGWB monitoring data

## 2.2 SDG 13 — Climate Action

- Groundwater depletion is a direct and measurable indicator of climate stress

- Long-run trend analysis (linear slope in m/year) reveals climate-driven depletion patterns

- Enables data-driven climate adaptation by identifying at-risk regions early

- Demonstrates responsible use of AI as a tool for environmental monitoring

# 3. Data Description

## 3.1 Source

Data was obtained from the Central Ground Water Board (CGWB) open dataset, specifically the district-wise quarterly well-level monitoring records made available through the India-WRIS portal and the GitHub repository maintained by Craig D'Souza (github.com/craigdsouza/cgwb).

## 3.2 Dataset Overview

| Attribute | Detail |
| --- | --- |
| Coverage Period | 1996 – 2017 (21 years) |
| Measurement Freq | Quarterly: Jan / May / Aug / Nov |
| Total Wells | ~180 monitoring wells across 6 districts |
| Measurement Unit | Metres below ground level (mbgl) |
| Key Variable | Mean depth to water table per district per quarter |
| Spatial Scope | 6 districts in Western Uttar Pradesh |

## 3.3 District Coverage

| District | No. of Wells | Data Points | Risk (Model Output) |
| --- | --- | --- | --- |
| Saharanpur | 24 | 142 | Low |
| Muzaffarnagar | 34 | 142 | Moderate |
| Meerut | 27 | 142 | Moderate |
| Bijnor | 30 | 141 | Low |
| Moradabad | 31 | 143 | Moderate |
| Bareilly | 32 | 145 | Low |

## 3.4 Data Cleaning

- Raw data arrived in wide format (one row per well, one column per quarterly period)

- Wells were filtered to the 6 target districts and melted to long format

- District-level mean depth computed per quarter across all active wells

- Small gaps (≤2 consecutive missing quarters) filled by linear interpolation

- Final output: 855 district-quarter rows across 6 districts

# 4. Methodology

## 4.1 Pipeline Overview

The project follows a 5-step machine-learning pipeline:

- Step 01 — Data Collection: Raw CGWB CSV download and district filtering

- Step 02 — Data Cleaning: Aggregation to district-level quarterly means, interpolation

- Step 03 — Feature Engineering: Lag variables + seasonal quarter index

- Step 04 — Model Training: Random Forest Regressor (primary) + Linear Regression (baseline)

- Step 05 — Risk Classification: Slope-based threshold labelling

## 4.2 Feature Engineering

The core insight is that groundwater depth follows seasonal and trend patterns that can be captured by the depth values in recent quarters. We created:

- lag_1 through lag_4: depth values from the previous 1–4 quarters

- quarter_of_year: the seasonal quarter (1–4) to capture monsoon/pre-monsoon variation

These 5 features make the model interpretable and directly relevant to the physical process — groundwater in the next season is predicted from the recent past plus seasonality.

## 4.3 Models

### 4.3.1 Random Forest Regressor (Primary Model)

- 300 decision trees, max depth 5, random state 42

- Trained on lag features and quarter-of-year index

- Recursive forward forecast: predicted value at t+1 fed back as lag input for t+2

- 12 quarters (3 years) forecasted forward from January 2017

- Validated via holdout test (last 8 quarters withheld, ~2 years)

### 4.3.2 Linear Trend Analysis (Interpretable Baseline)

- Simple Ordinary Least Squares regression: depth ~ time index

- Slope (m/quarter) converted to m/year by multiplying by 4

- Positive slope = water table declining (deeper)

- Used as the basis for risk level classification due to its interpretability

## 4.4 Risk Classification Thresholds

| Risk Level | Linear Slope Criterion | Interpretation |
| --- | --- | --- |
| High | ≥ 0.30 m/year | Rapid, sustained depletion — urgent intervention needed |
| Moderate | 0.10 – 0.29 m/year | Gradual depletion — monitoring and conservation planning required |
| Low | < 0.10 m/year | Stable or near-stable water table — continue monitoring |

# 5. Results

## 5.1 Model Performance (Holdout MAE)

Each model was evaluated by withholding the last 8 quarters (approximately 2 years) from training and computing Mean Absolute Error (MAE) on those held-out values:

| District | Trend (m/yr) | Risk Level | Holdout MAE (m) | Last Observed Depth (m) |
| --- | --- | --- | --- | --- |
| Saharanpur | +0.002 | Low | 0.544 m | 10.16 m |
| Muzaffarnagar | +0.216 | Moderate | 1.887 m | 13.20 m |
| Meerut | +0.125 | Moderate | 0.590 m | 9.62 m |
| Bijnor | +0.058 | Low | 0.771 m | 8.97 m |
| Moradabad | +0.134 | Moderate | 2.721 m | 10.44 m |
| Bareilly | +0.025 | Low | 0.368 m | 6.50 m |

## 5.2 Key Findings

- Muzaffarnagar shows the steepest long-run depletion at +0.216 m/year, the highest among all 6 districts

- Saharanpur is effectively stable at +0.002 m/year — nearly zero net change over 21 years

- Meerut and Moradabad are in the Moderate range, both declining at about 0.13 m/year

- Bareilly has the shallowest water table (6.50 m) and the lowest depletion rate (0.025 m/yr)

- All districts show clear seasonal oscillation (deeper in summer, shallower post-monsoon)

- 3 out of 6 districts classified as Moderate risk — warranting attention at policy level

# 6. Prototype — Groundwater Watch Dashboard

## 6.1 Overview

The project deliverable is a self-contained single-file HTML dashboard (groundwater_watch.html) that requires no internet connection, no server, and no installation. It opens directly in any modern browser by double-clicking the file.

## 6.2 Features

- District Selector: Switch between all 6 districts with one click

- Borehole Cross-section Visual: Animated graphic showing current water table depth in metres below ground level — designed to be intuitive for non-technical users such as farmers or local officials

- Risk Badge: Colour-coded Low / Moderate / High badge derived from the linear trend slope

- Trend Indicator: Shows the computed depletion rate in m/year

- Monitoring Wells Count: Displays number of CGWB wells used for that district

- Historical Trend Chart: Interactive Chart.js line chart of 20-year quarterly depth time series (1996–2017)

- 3-Year Forecast Line: Dashed line extending from 2017 to 2020 representing the Random Forest recursive forecast

- Offline & Portable: All data (855 historical data points + forecasts) are embedded as JSON in the HTML file

## 6.3 Technology Stack

| Component | Technology |
| --- | --- |
| Frontend | HTML5, CSS3, Vanilla JavaScript |
| Charts | Chart.js 4.4.1 (CDN) |
| Fonts | IBM Plex Sans, IBM Plex Mono, Spectral (Google Fonts) |
| Data Format | Embedded JSON (35KB compact) |
| ML Pipeline | Python 3.12, scikit-learn 1.x, pandas, numpy |
| Model | RandomForestRegressor (sklearn), LinearRegression (sklearn) |
| No. of Files | 1 HTML file (self-contained) |

## 6.4 How to Run

- Download groundwater_watch.html to any folder on your computer

- Double-click the file — it opens in Chrome, Firefox, Edge, or any modern browser

- Click any district button to switch the view — all charts and the borehole visual update instantly

- Works completely offline after download — no internet required

# 7. Responsible AI Considerations

## 7.1 Fairness

- Data covers all 6 districts with equal representation — no geographic bias in model training

- Risk labels use objective, explainable thresholds based on the physical depletion rate in m/year

- No demographic or personally identifiable data is used at any stage of the project

- No social group is disadvantaged by the model's outputs

## 7.2 Transparency

- Both models (Random Forest and Linear Trend) are fully documented in this report

- Holdout MAE values are published per district — no hidden accuracy claims

- The dashboard footer explicitly states the data time period and model scope

- Source code is structured and commented for full reproducibility

## 7.3 Ethics and Privacy

- All data used is publicly available government records (CGWB) — no private data involved

- The forecast is explicitly framed as an indicative trend, not a precise operational prediction

- The tool has no commercial purpose — it is a public-good academic project

- No risk of misuse as a surveillance or personal tracking tool

## 7.4 Limitations (Honest Disclosure)

- The data cuts off at January 2017 — post-2017 depletion trends are not captured or validated

- Some quarters have sparse well coverage (as few as 9–10 wells) which may affect district averages

- Random Forest with lag features may underestimate extreme depletion events not seen in training data

- The recursive forecast compounds prediction error over time — the 3-year projection should be treated as a directional trend, not a precise value

- Western UP coverage is good; this methodology should be re-validated before applying to other regions

# 8. Expected Impact

## 8.1 For Farmers

Farmers in Western UP depend on groundwater for pre-monsoon irrigation (rabi crop season). A district-level depletion forecast enables them to plan irrigation cycles and borewell usage before the growing season, potentially reducing water waste and improving yield planning.

## 8.2 For District Authorities and Jal Board UP

Moderate-risk districts like Muzaffarnagar, Meerut, and Moradabad can be prioritised for targeted groundwater recharge schemes, no-objection-certificate (NOC) restrictions on new borewells, and extraction permit planning. The risk classification provides a clear, defensible basis for such decisions.

## 8.3 For Environmental Planners and Policymakers

The depletion rate map across 6 districts provides a quantitative baseline for groundwater conservation zone designation, land-use planning decisions, and budget allocation for water conservation programmes at state level.

## 8.4 For Researchers

The pipeline (Python scripts + clean CSV + forecast JSON) is fully reproducible and can serve as a baseline model to be extended with post-2017 CGWB data, additional districts, or more advanced models (LSTM, SARIMA) as follow-on work.

# 9. File Structure & Deliverables

| File | Description |
| --- | --- |
| groundwater_watch.html | Self-contained offline dashboard — open in any browser |
| 01_data_prep.py | Python script: raw CGWB CSV → clean district quarterly CSV |
| 02_train_forecast.py | Python script: trains Random Forest & Linear model, outputs forecast JSON |
| district_quarterly_clean.csv | Cleaned dataset: 855 rows, 6 districts, 1996–2017 |
| forecast_results.json | Model output: historical + 3-year forecast per district with risk labels |
| Groundwater_Forecasting.pptx | 9-slide presentation covering all guideline-required sections |
| Project_Documentation.docx | This document — full project report |

# 10. Conclusion

This project demonstrates that AI can be applied responsibly and meaningfully to a real-world sustainability challenge — groundwater depletion in Western Uttar Pradesh — using publicly available data and interpretable machine learning techniques.

Key accomplishments:

- Processed 21 years of quarterly CGWB well data across 6 UP districts (~180 monitoring wells)

- Built a two-model pipeline (Random Forest + Linear Trend) producing 3-year depletion forecasts

- Derived objective, explainable risk levels (Low / Moderate / High) for each district

- Delivered a fully self-contained, offline-capable web dashboard requiring no installation

- Documented all limitations honestly in line with responsible AI principles

The tool aligns with SDG 6 and SDG 13, is grounded in the local geography of Western UP, and is built with clarity, reproducibility, and real-world usability in mind — reflecting the core values of the 1M1B AI for Sustainability Internship.

Badal Kumar  ·  Quantum University, Roorkee  ·  June 2026